GUIDE SPECIFICATION

Manufacturer:

Oldcastle BuildingEnvelope®

P.O. Box 629

Terrell, Texas 75160

Voice (972) 551-6100

SECTION 08 44 13 - GLAZED ALUMINUM CURTAIN WALL (RELIANCE - TC LT )

*************************************************************************************************

This guide specification has been prepared by Oldcastle BuildingEnvelope® in printed and electronic media as an aid to specifiers in preparing written construction documents for glazed aluminum curtain wall systems.

This section includes aluminum exterior curtain wall systems for multi-story cladding, shop fabricated, factory pre-finished. This section can be modified to incorporate louvers, granite, marble, or insulated infill panels.

Sections 01 40 00 are intended for use along with this section; coordinate requirements accordingly.

Sealants are referenced to Section 07 92 00, Sealants Sealers.

Glass and glazing are referenced to Section 08 80 00, Glass and Glazing.

Firestopping is referenced to Section 07 84 00.

Where a storefront, sloped glazing system, or skylight integrates with curtain wall system, carefully coordinate both sections to function together.

Refer to AAMA Curtain Wall Design Guide Manual.

Edit entire master to suit project requirements.  Modify or add items as necessary. Delete items which are not applicable.  Words and sentences within brackets [_____] reflect a choice to be made regarding inclusion or exclusion of a particular item or statement.  This section may include performance, proprietary, and descriptive type specifications.  Edit to avoid conflicting requirements.

Editor notes are included within the text of this section to assist the specifier in knowledgeable decision-making.

*************************************************************************************************

PART 1 - GENERAL

1.01           SUMMARY

A.       Related Documents:  Conditions of the Contract, Division 1 - General Requirements, and Drawings apply to Work of this Section.

*************************************************************************************************

Edit this paragraph to briefly describe the contents of the section.  After editing section, refer back to this paragraph to verify no conflicts exist.

*************************************************************************************************

B.       Section Includes:

1.        Aluminum curtain wall systems, complete with reinforcing, shims, anchors, and attachment devices.

2.        Accessories necessary to complete Work.

C.       Products Furnished But Not Installed Under this Section:  Inserts and anchoring devices that are to be built into structure.

*************************************************************************************************

This document incorporates CSI (Construction Specifications Institute) Manual of Practice principles of cross-referencing to Division 1 sections and other sections.  The cross references must be edited to retain only those other sections used.  Other guide specifications for Oldcastle BuildingEnvelope®, including those of skylights are available as follows:

Section 08 41 13      Aluminum-Framed Entrances and Storefronts

Section 08 41 26      All-Glass Entrances and Storefronts

Section 08 43 13      Aluminum-Framed Storefronts

Section 08 44 13      Glazed Aluminum Curtain Wall

Section 08 44 23      Structural-Sealant-Glazed Curtain Wall

Section 08 44 33      Sloped Glazing Assemblies

Section 08 51 13      Aluminum Windows

Section 08 63 00      Metal-Framed Skylights

*************************************************************************************************

D.       Related Sections:

- Section 01 43 39          Mockup

- Section 05 50 00          Metal Fabrications.

- Section 06 10 00          Rough Carpentry.

- Section 07 92 00          Joint Sealants.

- Section 08 42 20          Entrances.

- Section 08 45 00          Translucent Wall and Roof Assemblies.

- Section 08 51 00          Metal Windows.

- Section 08 71 00          Door Hardware.

- Section 08 81 00          Glass Glazing.

- Section 08 85 00          Glazing Accessories.

- Section 08 88 00          Special Function Glazing.

- Section 08 91 19          Fixed Louvers.

************************************************************************************************

List reference standards that are included within the text of this section.  Edit the following as required for project conditions.

*************************************************************************************************

1.02           REFERENCES

A.       Aluminum Association (AA):

1.        DAF-45                  Designation System for Aluminum Finishes.

B.       American Architectural Manufacturers Association (AAMA):

1.        CW-DG-1-96            Aluminum Curtain Wall Design Guide Manual.

2.        501.2                      Quality Assurance and Diagnostic Water Leakage Field Check of Installed  Storefronts, Curtain Walls and Sloped Glazed Systems.

3.        2605                       Voluntary Specification, Performance Requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

4.        611                         Voluntary Specification for Anodized Architectural Aluminum

5.        701/702                   Voluntary Specifications for Pile Weatherstripping and Replaceable Fenestration Weatherseals.

6.        CW-10                     Care and Handling of Architectural Aluminum From Shop to Site.

7.        TIR-A8                     Structural Performance of Composite Thermal Barrier Framing Systems.

C.       American National Standards Institute (ANSI):

1.        Z97.1                      Safety Glazing Materials Used in Buildings - Safety Performance Specifications and Methods of Testing

D.       American Society for Testing and Materials (ASTM):

1.        A36                         Standard Specification for Carbon Structural Steel.

2.        A123                       Standard Specification for Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products.

3.        A525                       Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) by the Hot-Dip Process, General Requirements

4.        A526                       Standard Specification for Sheet Steel, Zinc Coated (Galvanized) by the Hot-Dip Process, Commercial Quality.

5.        B209                       Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate.

6.        B221                       Standard Specification for Aluminum-Alloy Extruded Bars, Rods, Wire, Shapes, and Tubes.

7.        B308                       Standard Specification for Aluminum-Alloy 6061-T6 Standard Structural Profiles

8.        C716                       Standard Specification for Installing Lock-Strip Gaskets and Infill Glazing Materials.

9.        C920                       Standard Specification for Elastomeric Joint Sealants.

10.      E283                       Standard Test Method for Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors under Specified Pressure Differences Across the Specimen

11.      E330                      Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference.

12.      E331                      Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors and  Curtain Walls by Uniform Static Air Pressure Difference.

E.       Consumer Product Safety Commission (CPSC):

1.        16 CFR 1201           Safety Standard for Architectural Glazing Materials.

F.       Federal Specifications (FS):

1.        TT-P-645A               Primer, Paint, Zinc Chromate, Alkyd Type.

G.       Glass Association of North America (GANA):

1.                                      Glazing Manual.

H.       Steel Structures Painting Council (SSPC):

1.        SP2                        Hand Tool Cleaning.

2.        SP3                        Power Tool Cleaning.

3.        Paint 12                  Cold-Applied Asphalt Mastic (Extra Thick Film).

*************************************************************************************************

Use this article carefully; restrict statements to identify system performance requirements or function criteria only.  Delete paragraphs not appropriate to project.

Performance specifying permits system manufacturers the latitude to adjust or redesign proprietary systems to achieve specified requirements.  Rely on this article as the "anchor" for curtain wall system specifying and minimize the material and component statements so not to conflict with performance criteria.

Edit system requirements carefully and include only applicable performance criteria.  Make sure there is no conflict with proprietary information listed in part 2.

*************************************************************************************************

1.03   SYSTEM REQUIREMENTS

A.       General Standard:  In addition to requirements shown or specified, comply with applicable provisions of Aluminum Curtain Wall Design Guide Manual for design, materials, fabrication and installation of component parts.

B.       Design Requirements:

*************************************************************************************************

Retain appropriate first item below; delete the others.  Refer to AAMA "Aluminum Curtain Wall Design Guide Manual" for clarification of stick framed systems vs unit framed systems.  Members of stick framed systems are installed independently of each other and are not pre-assembled.  Unit framed systems are factory pre-assembled and installed as unit.

*************************************************************************************************

1.        Metal stick framed systems with interior and exterior exposed metal framing.

2.        Operable vent with sight line concealed from the exterior.  Perimeter members to be integral with window wall framing.

3.        System manufacturer shall provide low profile entrance frames as an integral part of the curtain wall system.

4.        System manufacturer shall provide curtain wall systems, including necessary modifications to meet specified requirements and maintaining visual design concepts.

5.        Fabricate glazing systems for exterior glazing at vision areas and exterior glazing at spandrel areas.

6.        Perimeter conditions shall allow for installation tolerances, expansion and contraction of adjacent materials, and sealant manufacturer's recommended joint design.

7.        Drawings are diagrammatic and do not purport to identify nor solve problems of thermal or structural movement, glazing, anchorage or moisture disposal.

8.        Requirements shown by details are intended to establish basic dimension of unit, sight lines and profiles of members.

9.        Do not assume glass, sealants, and interior finishes contribute to framing member strength, stiffness, or lateral stability.

10.      Attachment considerations are to take into account site peculiarities and expansion and contraction movements so there is no possibility of loosening, weakening or fracturing connection between units and building structure or between units themselves.

11.      Allow for expansion and contraction due to structural movement without detriment to appearance or performance.

12.      System shall drain to exterior face of wall, water entering joints and condensation occurring within system by drain holes and gutters of adequate size to evacuate water without infiltration to interior or the top of lower lites of glass.

13.      Provide concealed fastening.

14.      Metal faces are required to be visually flat under all lighting conditions, subject to acceptance of Architect.

15.      Use dense EPDM isolators to maintain adequate compression on glazing material.

16.      Provide uniform color and profile appearance at components exposed to view.

17.      Provide interior dense EPDM gasket with sealed corners, with maximum 30% compression when glazed, to create a water and air seal. Provide exterior dense EPDM wedge gasket at the verticals and exterior EPDM gasket at the horizontals, with a maximum 30% compression when glazed, to create a water & air seal.

18.      Provide pre-punched pressure plates to ensure correct quantity and spacing of fasteners.

*************************************************************************************************

Retain item below when structural silicone glazing is utilized.

*************************************************************************************************

19.      Stresses placed on structural silicone sealants shall be kept within sealant manufacturer's recommended maximum.

20.      Not Permitted:  Vibration harmonics, wind whistles, noises caused by thermal movement, thermal movement transmitted to other building elements, loosening, weakening, or fracturing of attachments or components of system.

*************************************************************************************************

Coordinate performance requirements with section 01 40 00.

*************************************************************************************************

C.       Performance Requirements:

1.    Air infiltration:  Air leakage shall not exceed 0.06 cfm per square foot of surface area when tested in accordance with ASTM E283 at differential static pressure of 6.24 psf (300 Pa).

2.    Water Resistance (static):  No uncontrolled leakage when tested in accordance with ASTM E331 at test pressure of 20 psf (960 Pa) as defined in AAMA 501.

3.    Water Resistance (cyclic): No uncontrolled leakage when testing in accordance with ASTM E547 at a test pressure of 20 psf (960 Pa) as defined in AAMA 501.

4.    Water Resistance (severe, wind driven rain): No uncontrolled leakage when tested in accordance with ASTM E2268 and AAMA 520 at level 10, pressure of 14 psf (670 Pa) to 42 psf (2010 Pa).

5.    Water Resistance (dynamic):  No uncontrolled leakage when tested in accordance with ASTM E331 at test pressure of 20 psf (960 Pa) as defined in AAMA 501.

6.    Uniform Load:  A static air design load of 40 psf (1915 Pa) shall be applied in a positive and negative direction in accordance with ASTM E 330.  At structural test load equal to 1.5 times the specified design load, no glass breakage or permanent set in the framing members in excess of 0.2% of their clear spans shall occur.

7.    Seismic: Must meet design displacement (elastic) of 0.010 times the story height and ultimate displacement (inelastic) of 1.5 times the design displacement tested in accordance to AAMA 501.4.

8.    Seismic: Must meet dynamic seismic drift causing glass cracking (∆Cracking) of 3” or 0.0300 times the story height, and glass fallout (∆Fallout) of 5” or 0.0500 times the story height tested in accordance to AAMA 501.6.

*************************************************************************************************

Coordinate wind loads with applicable building code, or appropriate wind loads may be determined by using ANSI A58.1, "Minimum Design Loads for Buildings and Other Structures".

Boundary layer wind tunnel testing may be necessary for determining design wind loads when building shape is other than rectangular in plan, site location has unusual wind conditions, or building is critically located with respect to other nearby buildings.

Edit following paragraph accordingly.  Coordinate provisions with Section 01 40 00.

*************************************************************************************************

D.       Structural Requirements:

1.        Wind loading:

a.       Basic zones:

b.       Corner zones:

[c.      Parapet zones:]

2.        Deflection under uniform loading:  When tested in accordance with ASTM E330 at design pressure, maximum deflection of exterior member shall not exceed L/175 for spans up to 13’-6” or L/240 + 1/4” for spans greater than 13’-6”.

3.        Parallel to wall and corner mullion deflections:  75% of glass edge bite or 3/8 inch, whichever is less.

4.        Compression flanges of flexural members may be assumed to receive effective lateral bracing only from:

a.       Anchors to building structure and

b.       Horizontal glazing rails or interior trim, which are in actual contact with compression flange.

5.        Do not regard points of contra-flexture as lateral braces or as end points of un-braced length; un-braced length is actual distance between effective lateral braces as defined above.

- Thermal Requirements:  Framing systems shall accommodate expansion and contraction movement due to surface temperature differential of 180°F without causing buckling, stress on glass, failure of joint seals, excessive stress on structural elements, reduction of performance or other detrimental effects in accordance with AAMA 505.

- Thermal Transmittance (U-factor):  When tested to AAMA Specification 1503:

- Aluminum Pressure Plate: The thermal transmittance (U-Factor) shall not be more than 0.29 Btu/h-ft²  -F (2.38 W/m²-K) (with Low-E glass e=0.021).

- Polyamide Pressure Plate: The thermal transmittance (U-Factor) shall not be more than 0.28 Btu/h-ft²  -F (2.38 W/m²-K) (with Low-E glass e=0.021).

- Condensation Resistance (CRF):  When tested to AAMA Specification 1503, the condensation resistance factor shall not be less than 68 (for the frame).

- Vertical Interstory Movement:  When tested to AAMA 501.7, system must meet an interstory drift of  ±1/2".

- Sound Transmission:

1.        When tested to ASTM E90-09, the Sound Transmission Class (STC) shall not

be less than 37 based upon 1” (25mm) insulating laminated glass (1/4” (6mm)

laminate, 1/2” (12.7mm) air space,1/4” (6mm) laminate).

2.        When tested to ASTM E90-09, the Sound Transmission Class (STC) shall not

be less than 32 based upon 1” (25mm) insulating  glass (1/4” (6mm) clear

tempered, 1/2”  (12.7mm) air space, 1/4” (6mm) clear tempered.)

K.       Outdoor Indoor Transmission:

1.        When tested to ASTM E90-09, the Outdoor Indoor Transmission Class (OITC)

shall not be less than 31 based upon 1” (25mm) insulating laminated glass (1/4”

(6mm) laminate, 1/2”(12.7mm) air space,1/4” (6mm) laminate).

2.        When tested to ASTM E90-09, the Outdoor Indoor Transmission Class (OITC)

shall not be less than 27 based upon 1” (25mm) insulating  glass (1/4” (6mm)

clear tempered, 1/2” (12.7mm) air space, 1/4” (6mm) clear tempered.)

upon 1” insulating glass (1/4” Lam, 1/2” AS, 1/4” Lam).

[L.      Laboratory Testing:  Refer to Section 01 40 00 for requirements.]

M.      Interface:

1.        Furnish inserts and anchoring devices, which need to be preset and built into structure to appropriate trade.

2.        Supply on timely basis to avoid delay in Work.

3.        Instruct other trades of proper location and position.

4.        Furnish setting drawings, diagrams, templates and installation instructions.

*************************************************************************************************

Include submittal requirements below, which are consistent with scope of project and extent of work of this section.  Only request submittals that are absolutely necessary.

*************************************************************************************************

1.04           SUBMITTALS

A.       General:  Submit in accordance with Section 01 33 00.

B.       Product Data:

1.        Submit manufacturer's descriptive literature for each manufactured products.

2.        Include information for factory finishes, accessories and other required components.

3.        Declare Label - Documentation reporting a complete list of chemical ingredients to at least 100 ppm (0.01%) that covers 100% of the product.

*************************************************************************************************

Retain item below when manufacturer's standard colors are specified for pigmented finishes or recycled content is required, coordinate with finish article.

*************************************************************************************************

[4.       Include color charts for finish indicating manufacturer's standard colors available for selection.]

[5.       Provide documentation confirming aluminum has a minimum of 50% recycled content.]

C.       Shop Drawings:

1.        Submit drawings indicating elevations, detailed design, dimensions, member profiles, joint locations, arrangement of units, member connections, and thickness of various components.

2.        Show following items:

a.       Details of special shapes.

b.       Reinforcing.

c.       Drainage details and flow diagrams.

d.       Anchorage system.

e.       Interfacing with building construction.

f.        Provisions for system expansion and contraction

[g.      Thermal breaks.]

4.        Indicate glazing details, methods, [locations of various types and thickness of glass, emergency breakout locations,] and internal sealant requirements.

5.        Clearly indicate locations of exposed fasteners and joints for Architect's acceptance.

6.        Clearly show where and how manufacturer's system deviates from Contract Drawings and these Specifications.

[D.      Mock-up Drawings:  Submit drawings for mock-ups; refer to Section 01 43 39 for mock-up requirements.]

*************************************************************************************************

Retain data within brackets in first subparagraph when pigmented finish is specified; coordinate with article 2.07.

*************************************************************************************************

E.       Samples:

1.        Submit manufactures samples indicating quality of finish in required colors.

2.       Where normal texture or color variations are expected, include additional samples           illustrating range of variation.

[3.       Submit samples of structural glazing gaskets, 12 inch lengths.]

[4.       Submit samples of sealants for color selection.]

***** OR *****

*************************************************************************************************

Following paragraph permits option to submit results of pre-tested existing fixed window designs.  Retain when applicable.

*************************************************************************************************

[F.      Test Reports:  Submit certified copies of previous tests reports by independent laboratory substantiating performance of system.  Include other supportive data as necessary.]

*************************************************************************************************

Retain data within brackets in paragraph below for structural silicone glazed system.

*************************************************************************************************

I.        Manufacturer's Instructions:  Submit manufacturer's printed installation instructions.  [Include detailed instructions describing each step of re-glazing procedures.]

J.       Warranty:  Submit specified warranties.

*************************************************************************************************

Include quality assurance requirements consistent with size and scope of project and extent of work of this section.  Edit following article accordingly.

*************************************************************************************************

1.05   QUALITY ASSURANCE

*************************************************************************************************

Oldcastle BuildingEnvelope® is unique in the industry in single source responsibility.  First, system design, extrusion, fabrication, and finishing occur at the same facility, and under strict tolerances, assuring uniformity of profile and finishes between systems.  Second, Oldcastle BuildingEnvelope® products include a full array of storefront, curtain wall, all glass entrances, sliding mall fronts, sloped glazing, and flush faced aluminum framed doors, as well as all the monumental and unit skylight products, allowing the designer and specifier a single source of responsibility when combining products from any of these categories.

*************************************************************************************************

A.       Single Source Responsibility:

1.        Provide curtain wall systems that are products of a single manufacturer.

B.       Engineer Qualifications:  Professional Structural Engineer registered in State where Project is located.

C.       Installer Qualifications:  Certified in writing by system manufacturer as qualified for specified systems.

*************************************************************************************************

Depending on scope of work, mock-ups may not be required; retain and edit following article accordingly.  Ensure section 01 43 39 includes details for each mock-up required.

*************************************************************************************************

*************************************************************************************************

Depending on scope of work, pre-installation conference may not be required.  Retain article below when applicable and edit accordingly.

*************************************************************************************************

[1.06          PRE-INSTALLATION CONFERENCE

A.       Conduct pre-installation conference in accordance with Section 01 31 19.13.

*************************************************************************************************

Retain paragraph below when structural silicone glazing is required.

*************************************************************************************************

[B.      Conference Purpose and Agenda:

1.        Arrange with Architect and representatives of window and sealant manufacturer to visit Project site [factory] before beginning glazing operations to analyze site conditions, and inspect surfaces and joints to be sealed in order that recommendations may be made should adverse conditions exist.

2.        Discuss following items:

a.       Weather conditions under which work will be done.

b.       Anticipated frequency and extent of joint movement.

c.       Joint design.

d.       Glazing procedures.]

1.07           DELIVERY, STORAGE, AND HANDLING

A.       Comply with requirements of Section 01 65 00 - 01 66 00.

B.       Protect finished surfaces to prevent damage.

C.       Do not use adhesive papers or sprayed coatings, which become firmly bonded when exposed to sun.

D.       Do not leave coating residue on surfaces.

*************************************************************************************************

Retain two following paragraphs when glass is included as work of this section.

*************************************************************************************************

[E.      Deliver glass units with manufacturer's labels intact on interior side of glass.  Ensure labels indicate glass thickness, unit location, glass strength and orientation of units in vertical position.]

[F.      Protect glass edges and corners to prevent chipping, cracking, and other similar damages.]

1.08           PROJECT CONDITIONS

A.       Ensure ambient and surface temperatures and joint conditions are suitable for installation of materials.

1.09              WARRANTY

A.       Provide warranties in accordance with Section 01 78 36.

B.       Provide written warranty in form acceptable to Owner jointly signed by manufacturer, installer and Contractor warranting work to be watertight, free from deflective materials, defective workmanship, glass breakage due to defective design, and agreeing to replace components which fail within 1 year from date of Substantial Completion.

C.       Warranty shall cover following:

1.        Complete watertight and airtight system installation within specified tolerances.

2.        Glass and glazing gaskets will not break or "pop" from frames due to design wind, expansion or contraction movement or structural loading.

3.        Glazing sealants and gaskets will remain free from abnormal deterioration or dislocation due to sunlight, weather or oxidation.

*************************************************************************************************

Delete paragraph below if high performance fluoropolymer finish not used.

*************************************************************************************************

D.       Provide written warranty stating organic coating finish will be free from fading more than 10%, chalking, yellowing, peeling, cracking, pitting, corroding or non-uniformity of color, or gloss deterioration beyond manufacturer's descriptive standards for 5 years from date of Substantial Completion and agreeing to promptly correct defects.

PART 2 - PRODUCTS

2.01           MANUFACTURERS AND PRODUCTS

*************************************************************************************************

In this article, list the manufacturers acceptable for this project.

*************************************************************************************************

A.       Subject to compliance with requirements indicated, provide products by one of the following:

1.        Oldcastle BuildingEnvelope®, Terrell, TX.

B.       Substitutions: Submit under provisions of Section 01 63 00, a minimum of 10 days prior to bid date.

*************************************************************************************************

Edit the following paragraphs for appropriate system in each category and delete remaining.  Refer to Oldcastle BuildingEnvelope® technical literature for additional information.

When specifying manufacturer's standard product or manufacturer's standard product with modifications, describe using manufacturer's name and model numbers.

*************************************************************************************************

Oldcastle BuildingEnvelope® curtain wall systems included in this section are as follows:

RELIANCE-TC LT CURTAIN WALL -

1" Glazing:    2" x 6-1/4", or 7-1/2" mullion profiles; pressure glazed, front set, exterior glazed, stick wall system using Polyamide Pressure Plate.

1” systems is available with structural silicone glazing (SSG) option.

*************************************************************************************************

2.02           FRAMING MATERIALS AND ACCESSORIES

A.       Aluminum:

1.        ASTM B221, alloy 6063-T6 for extrusions; ASTM B209, alloy 5005-H16 for sheets; or other alloys and temper recommended by manufacturer appropriate for specified finish.

B.       Internal Reinforcing:

1.        ASTM A36 for carbon steel; or ASTM B308 for structural aluminum.

2.        Shapes and sizes to suit installation.

3.        Shop coat steel components after fabrication with alkyd type zinc chromate primer complying with FS TT-P-645.

C.       Inserts and Anchorage Devices:

1.        Manufacturer's standard formed or fabricated assemblies, steel or aluminum, of shapes, plates, bars or tubes.

2.        Hot-dip galvanize steel assemblies after fabrication, comply with ASTM A123, 2.0 ounce minimum coating.

***** OR *****

[3.       Shop coat steel assemblies after fabrication with alkyd type zinc chromate primer complying with FS TT-P-645.]

D.       Fasteners:

1.        Non-magnetic stainless steel or cadmium plated steel coated with yellow or silver iridescence plating, compatible with materials being fastened.

2.        Series 300 stainless steel for exposed locations.  Cadmium plated steel with 0.0005 inch plating thickness and color chromate coated for concealed locations.

3.        Provide nuts or washers of design having the means to prevent disengagement; deforming of fastener threads is not acceptable.

4.        Provide concealed fasteners wherever possible.

5.        For exposed locations, provide countersunk flathead fasteners with finish matching item fastened.

E.       Expansion Anchor Devices:  Lead-shield or toothed-steel, drilled-in, expansion bolt anchors.

F.       Shims:  Non-staining, non-ferrous, type as recommended by system manufacturer.

G.       Protective Coatings:  Cold applied asphalt mastic complying with SSPC-Paint 12, compounded for 30 mil thickness for each coat; or alkyd type zinc chromate primer complying with FS TT-P-645.

H.       Glazing Gaskets:

1.        Compression type design, exterior replaceable, extruded EPDM.  Interior is a dense EPDM gasket.

2.        Comply with ASTM C509 or C864.

3.        Profile and hardness as necessary to maintain uniform pressure for watertight seal.

4.        Manufacturer's standard black color.

I.        Internal Sealants:  Types recommended by system manufacturer to remain permanently non-hardening, non-migrating and weather-tight.

[J.      Curtain Wall Insulation and Fire-Safing:  Refer to Sections 07 21 00 and 07 27 00.]

*************************************************************************************************

Retain paragraph below if architectural aluminum panels are required in curtain wall.

*************************************************************************************************

[K.      Spandrel Panels and Exterior Column Covers [Soffits and Metal Ceilings]:

1.        Type:  Aluminum sheet, 1/8 inch thick, suitably reinforced on concealed surface for surface flatness, or prefabricated sandwich panels at manufacturer's option.

2.        Surface flatness:  0.015 inch maximum deviation when measured with 6 inch rule.

3.        Squareness:  0.002 inch maximum for each inch of length at panel edge.

4.        Anchorage:  Allow for expansion and contraction, to minimize oil canning and distortion.]

*************************************************************************************************

Retain paragraph below if architectural aluminum sunshades are required on curtain wall.

*************************************************************************************************

[L.      Sunshades:

1.        Type:  Extruded aluminum anchors, outriggers, louvers, and fascia.

2.        Shapes:  May be chosen from manufacturer’s standard shapes shown on website or in Detail Catalog.

3.        Engineering:  Manufacturer to verify sufficient strength of wall system to support loads imposed by the sunshade.]

2.03           GLASS AND GLAZING ACCESSORIES

A.       Refer to Section 08 85 00.

2.04           SYSTEM FABRICATION

A.       Take accurate field measurements to verify required dimensions prior to fabrication.

B.       Location of exposed joints is subject to Architect's acceptance.

C.       Provide dense EPDM continuous isolator to separate exterior pressure plates and interior framing members.

D.       Fabricate components in accord with approved shop drawings.  Remove burrs and ease edges.  Shop fabricate to greatest extent practicable to minimize field cutting, splicing, and assembly.  Disassemble only to extent necessary for shipping and handling limitations.

E.       Steel Components:

1.        Clean surfaces after fabrication and immediately prior to application of primer in accord with SSPC-SP2 or SSPC-SP3 at manufacturer's option.

2.        Apply specified shop coat primer in accord with manufacturer's instructions to provide 2.0 minimum dry film thickness.

F.       Fabricate components true to detail and free from defects impairing appearance, strength or durability.  [Fabricate custom extrusions indicated and as necessary for complete installation.]

G.       Fabricate components to allow for accurate and rigid fit of joints and corners.  Match components carefully ensuring continuity of line and design.  Ensure joints and connections will be flush and weather-tight.  Ensure slip joints make full, tight contact and are weather-tight.

H.       Reinforce components as required at anchorage and support points, at joints, and at attachment points for interfacing work.

I.        Provide structural reinforcing within framing members where required to maintain rigidity and accommodate design loads.

J.       System design and sealants to accommodate internal weep and drainage system not visible to the exterior.

K.       Head and sill extrusions act as gutter and weep water to exterior; do not penetrate sections with fasteners.

L.       Allow for adequate clearance around perimeter of system to enable proper installation and for thermal movement within system.

M.      Separate dissimilar metals with protective coating or preformed separators to prevent contact and corrosion.

*************************************************************************************************

Retain paragraph below if solid spandrel and column infill panels are required.

*************************************************************************************************

[N.      Provide framing members to rigidly glaze spandrel panels and column covers within framing system.]

[O.      Provide special shapes and filler pieces with tight corners.]

*************************************************************************************************

Select and edit following items for appropriate finish; delete inapplicable types.  Oldcastle BuildingEnvelope® is a licensed applicator for all of the coating manufacturers listed below.

*************************************************************************************************

2.05         FINISH

A.       Organic Coating (high performance fluorocarbon):

1.        Comply with requirements of AAMA 2605.

2.        Surfaces cleaned and given conversion coating pre-treatment prior to application of 0.3 mil dry film thickness of epoxy or acrylic primer following recommendations of finish coat manufacturer.

*************************************************************************************************

Note:  A less expensive finish coat containing a minimum of 50% fluorocarbon resin is also available, and meets AAMA 2605, but with reduced performance over time.

*************************************************************************************************

3.        Finish coat of [50%] [70%] minimum fluorocarbon resin fused to primed surfaces at temperature recommended by manufacturer, 1.0 mil minimum dry film thickness.

4.        Acceptable coatings are Trinar by Akzo Coatings, Inc.; Nubelar by Glidden Company; Fluoroceram by Morton International, Inc.; Duranar by PPG Industries Inc.; and Fluropon by Valspar Corporation.

5.        Provide in either 2, 3, or 4 coat system as required for color selected.

[6.       Custom colors as selected by Architect.]

***** OR *****

[7.       Manufacturer's standard colors as selected by Architect.]

***** OR *****

*************************************************************************************************

Oldcastle BuildingEnvelope® utilizes a computer driven anodizing system  which produces the closest color range available.

*************************************************************************************************

[B.      Clear Anodized:

1.        Conforming to AA-M12C22A31 and AAMA 611.

2.        Architectural Class I, etched, medium matte, clear anodic coating, 0.7 mil minimum thickness.]

***** OR *****

[C.      Color Anodized:

1.        Conforming to AA-M12C22A42 or A43 or A44 and AAMA 611

2.        Architectural Class [I], etched, medium matte, [black] [dark bronze] [medium bronze] [light bronze] colored anodic coating, [0.7].

PART 3 - EXECUTION

3.01         EXAMINATION

A.       Examine conditions and proceed with Work in accordance with Section 01 40 00.

B.       Verify dimensions, tolerances, and method of attachment with other Work.

3.02         INSTALLATION

A.       Install in accordance with manufacturer's instructions and applicable provisions of AAMA Aluminum Curtain Wall Design Guide Manual.

B.       Align assemblies plumb and level, free of warp or twist, aligning with adjacent Work.

C.       Tolerances:

1.        Limit variations from plumb and level:

a.       1/8 inch in 20'-0" vertically and horizontally.

b.       1/4 inch in 40'-0" either direction.

2.        Limit offsets in theoretical end-to-end and edge-to-edge alignment:

a.       1/16 inch where surfaces are flush or less than 1/2 inch out of flush and separated by not more than 2 inches.

b.       1/8 inch for surfaces separated by more than 2 inches.

3.        Step in face:  1/16 inch maximum.

4.        Jog in alignment:  1/16 inch maximum.

5.        Location:  1/4 inch maximum deviation of any member at any location.

6.        Tolerances are not accumulative.

D.       Provide attachments and shims to permanently fasten system to building structure.

E.       Anchor securely in place, allowing for required movement, including expansion and contraction.

F.       Separate dissimilar materials at contract points, including metal in contact with masonry or concrete surfaces, with protective coating or preformed separators to prevent contact and electrolytic action.

- G.       Set sill members in bed of sealant.  Set other members with internal sealants and baffles to provide weather-tight construction.

- H.       Water Drainage:  Each light of glass shall be compartmentalized using joint plugs and silicone sealant to divert water to the horizontal weep locations.  Weep holes shall be located in the horizontal pressure plates and covers to divert water to the exterior of the building.

*************************************************************************************************

Retain paragraph below when applicable.

*************************************************************************************************

[I.       Do not apply mullion covers until building is closed in, roofing is installed and no alkaline substances can be washed from building onto curtain wall system.]

*************************************************************************************************

Retain following paragraph when curtain walls are installed adjacent to stone.

*************************************************************************************************

[J.      Ensure that dead-load from curtain wall system is not transferred to stone veneer.]

K.       Glazing:

- Install glazing gaskets and sealants in accordance with manufacturer's instructions without exception; including surface preparations.  Refer to Section 08810 for additional requirements.

- With extruded aluminum pressure plates anchored to the mullion with 1/4" stainless steel bolts no greater than 9" on center “or” polyamide pressure plates anchored no greater than 6" on center.

L.       Fire-Safing and Curtain Wall Insulation:

1.        Install fire-safing and curtain wall insulation specified in Section 07 21 00 and 07 27 00.

*************************************************************************************************

Delete article below if not applicable.

*************************************************************************************************

3.03         FIELD QUALITY CONTROL

*************************************************************************************************

Edit paragraph below to include required field tests; air and water infiltration tests usually not required.  Coordinate provisions with referenced section.

*************************************************************************************************

A.       Field Tests:  Independent testing laboratory will perform [air infiltration,] [water infiltration,] [and] hose test; refer to Section 01 43 00 for requirements.

3.04         CLEANING

A.       Clean surfaces in compliance with manufacturer's recommendations; remove excess mastic, mastic smears, and other foreign materials.

B.       Clean metal surfaces exercising care to avoid damage.

END OF SECTION